40G-AOC-A1-XXM

40Gb/s QSFP+ TO QSFP+ Active Optical Cable

Features

- Support 40GBASE-SR4/QDR application

- Compliant to QSFP+ Electrical MSA SFF-8436

- Multi rate of up to 10.3125Gbps

	+3.3V single power supply

- Low power consumption

- Operating case temp Commercial: 0°C to +70 °C

- RoHS compliant

Applications

- 40GBASE-SR4 at 10.3125Gbps per lane

- InfiniBand QDR

- Other optical links

Absolute Maximum Ratings

Table2- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc3 | -0.5 | - | +3.6 | V |  |
| Storage Temperature | Ts | -10 | - | +70 | °C |  |
| Operating Humidity | RH | +5 | - | +85 | % | 1 |

Note: 1	No condensation

Recommended Operating Conditions

Table 3- Recommended operating Conditions

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |
| Power Supply Voltage | Vcc | 3.14 | 3.3 | 3.47 | V |  |

| Power Dissipation | Pd | - | - | 1.5 | W | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Bit Rate | BR | 1.25 | 10.3125 | - | Gbps |  |

Note: 1	Per terminal

Electrical Characteristics

Table 4- Electrical Characteristics

| Parameter |  | Symbol | Min. | Typ. | Max. | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ModSelL | Module Select | VOL | 0 | - | 0.8 | V |  |
|  | Module Unselect | VOH | 2.5 | - | VCC | V |  |
| LPMode | Low Power Mode | VIL | 0 | - | 0.8 | V |  |
|  | Normal Operation | VIH | 2.5 | - | VCC+0.3 | V |  |
| ResetL | Reset | VIL | 0 | - | 0.8 | V |  |
|  | Normal Operation | VIH | 2.5 | - | VCC+0.3 | V |  |
| ModPrsL | Normal Operation | VOL | 0 | - | 0.4 | V |  |
| IntL | Interrupt | VOL | 0 | - | 0.4 | V |  |
|  | Normal Operation | VoH | 2.4 | - | VCC | V |  |
| Electrical transmitter Characteristics |  |  |  |  |  |  |  |
| Differential Date Input Swing |  | Vout | 200 | - | 1600 | mV |  |
| Output Differential Impedance |  | ZD | 90 | 100 | 110 | Ω |  |
| Electrical Receiver Characteristics |  |  |  |  |  |  |  |
| Differential Data Output Swing |  | Vin,P-P | 350 | - | 800 | mVPP |  |
| Bit Error Rate |  | BER |  |  | E-12 |  | 1 |
| Input Differential Impedance |  | ZIN | 90 | 100 | 110 | Ω |  |

Note: 1	PRBS2^31-1@10.3125Gbps

Recommended Interface Circuit

Figure 1, Recommended Interface Circuit

Pin arrangement

Table 5-Pin Function Definitions

Figure 2, Pin View

| Pin | Symbol | Name/Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground | 1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 | GND | Ground | 1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 | GND | Ground | 1 |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | Vcc Rx | +3.3V Power Supply Receiver |  |
| 11 | SCL | 2-wire serial interface clock |  |
| 12 | SDA | 2-wire serial interface data |  |
| 13 | GND | Ground | 1 |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |
| 16 | GND | Ground | 1 |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output |  |
| 19 | GND | Ground | 1 |
| 20 | GND | Ground | 1 |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 | GND | Ground | 1 |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Ground | 1 |

| 27 | ModPrsL | Module Present |  |
| --- | --- | --- | --- |
| 28 | IntL | Interrupt |  |
| 29 | Vcc Tx | +3.3V Power supply transmitter |  |
| 30 | Vcc1 | +3.3V Power supply |  |
| 31 | LPMode | Low Power Mode |  |
| 32 | GND | Ground | 1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground | 1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground | 1 |

Note: 1. Circuit ground is internally isolated from chassis ground.

Monitoring Specification

Figure 3, Memory Map

Mechanical

Unit mm

|  | L | L1 | L2 | L3 | L4 | W | W1 | W2 | H | H1 | H2 | H3 | H4 | H5 | H6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max | 72.2 | - | 128 | 4.35 | 61.4 | 18.45 | - | 6.2 | 8.6 | 12.4 | 5.35 | 2.5 | 1.6 | 2.0 | - |
| Type | 72.0 | - | - | 4.20 | 61.2 | 18.35 | - | - | 8.5 | 12.2 | 5.2 | 2.3 | 1.5 | 1.8 | 6.55 |
| Min | 68.8 | 16.5 | 124 | 4.05 | 61.0 | 18.25 | 2.2 | 5.8 | 8.4 | 12.0 | 5.05 | 2.1 | 1.3 | 1.6 | - |

Table 6- Cable Length

Figure 4, Mechanical Diagram

| Cable Length（Unit: m） | Tolerant（Unit: cm） |
| --- | --- |
| ＜1.0 | +5/-0 |
| 1.0~4.5 | +15/-0 |
| 5.0~14.5 | +30/-0 |
| ≥15.0 | +2%/-0 |

Revision history

| Version | Initiated | Reviewed | Revision | Release Date |
| --- | --- | --- | --- | --- |
| A0 | Ling | Zhang | New Release | 2018-6-1 |
| A1 | Ling | Zhang | Mechanical Change | 2018-12-28 |